Cruise Control

Software Requirements

Authors	Requirements Team

Last Update	May 11th, 2022

Summary

This document defines requirements of a cruise control system for an automatic transmission car.

Table Of Contents

1.	Introduction	3

2.	Cruise Control Requirements	3

2.1	Interface	3

2.2	Cruise Control behavior	4

2.3	Car driving control	5

2.4	Cruise speed management	5

2.5	Pedals pressed detection	6

2.6	Cruise Control parameters	6

# Introduction

The Cruise control is a system to automatically control the speed of a car. The driver sets the speed, and the system will take over the throttle of the car to maintain the same speed.

# Functional Requirements

## Interface

The Cruise Control (CC) connects to the system using the following inputs:

- on (bool): Enable the CC

- off (bool): Disable the CC

- resume_ (bool): Resume the CC after a braking

- set (bool): Set the current speed as the new cruise speed

- quickDecel (bool): Decrease the cruise speed

- quickAccel (bool): Increase the cruise speed

- accel (float32, percentage): Accelerator pedal sensor

- brake (float32, percentage): Brake pedal sensor

- speed (float32, km/h): Car speed sensor

The CC provides the followings outputs:

- cruiseSpeed (float32, km/h): Cruise speed value

- throttleCmd (float32, percentage): Throttle command

- cruiseState (OFF, ON, STDBY or INT): CC’s state

## Cruise Control Behavior

CC_HLR_CCB_01

When the driver starts the car, the CC shall be off.

The output cruiseState should be set to OFF.

CC_HLR_CCB_02

The CC shall be on when the driver pushes the On button.

The output cruiseState shall be different from OFF e.g., one of ON, STDBY or INT.

CC_HLR_CCB_03

The CC shall be off when the Off button is pressed.

The output cruiseState shall be set to OFF.

CC_HLR_CCB_04

If the car speed is in the speed limit and the accelerator and brake pedals are not pressed, the CC shall be on and regulate the car speed.

The output cruiseState shall be set to ON.

CC_HLR_CCB_05

The CC system shall be automatically off when the accelerator pedal is pressed, or the car speed is outside the speed limit.

The output cruiseState shall be set to STDBY.

CC_HLR_CCB_06

The system shall return to the ON state when both the accelerator pedal is not pressed, and the car speed is inside the speed limits.

The output cruiseState shall be set to ON.

CC_HLR_CCB_07

The CC shall be immediately interrupted when the brake is pressed.

The output cruiseState shall be set to INT.

CC_HLR_CCB_08

The system shall resume either to the ON or STDBY states, depending on the accelerator pedal and the speed value when the resume button is pressed.

## Car Driving Control

CC_HLR_CDC_01

When the CC is off, the car speed shall be driven using the accelerator pedal.

CC_HLR_CDC_02

When the CC is on, the car speed shall be automatically regulated.

CC_HLR_CDC_03

The regulation shall be done using a proportional and integral algorithm, with KP and KI factors.

CC_HLR_CDC_04

The regulation algorithm shall be protected against the overshoot of its integral part: the integral action shall be reset when the CC is going on, and frozen when the throttle output is saturated.

CC_HLR_CDC_05

The throttle command shall be saturated at THROTTLE_SAT_MAX when automatically regulating, to limit the car acceleration for the comfort.

## Cruise Speed Management

CC_HLR_CSM_01

The cruise speed shall be managed only when the cruise control is enabled, meaning ON, STDBY, or INT states.

CC_HLR_CSM_02

The cruise speed shall be set to the current speed when the set button is pressed.

CC_HLR_CSM_03

The cruise speed shall be increased of SPEED_INC when the quickAccel button is pressed, with respect to SPEED_MAX.

CC_HLR_CSM_04

The cruise speed shall be decreased of SPEED_INC when the quickDecel button is pressed, with respect to SPEED_MIN.

CC_HLR_CSM_05

The cruise speed shall be maintained between SPEED_MIN and SPEED_MAX.

CC_HLR_CSM_06

The cruise speed shall remain identical when none of the buttons set, quickAccel and quickDecel are pressed.

## Pedals pressed detection

CC_HLR_PPD_01

The accelerator pedal shall be detected as pressed when its value is above PEDALS_MIN.

CC_HLR_PPD_02

The brake pedal shall be detected as pressed when its value is above PEDALS_MIN.

## Cruise Control parameters

- SpeedMin: 30.0_f32 km/h

- SpeedMax: 150.0_f32 km/h

- SpeedInc: 2.5_f32 km/h

- Kp: 8.113_f32

- Ki: 0.5_f32

- ThrottleSatMax: 100.0_f32 percent

- PedalsMin: 3.0_f32 percent.

# Architecture Notes

The cruise control interface has been introduced in the section 2.1, Interface.

When the cruise control is enabled, a first part computes the target cruise speed, and the other one ensures the regulation:

The function CruiseRegulation is part of RegulationMgt and implements the algorithm described in CC_HLR_CDC_03.